3.1 Data Collection and Acquisition

This study utilized a suite of physical, geomorphic, environmental, and socioeconomic datasets to support the GIS, GEE, and ML-based analyses. All datasets were standardized to a 30 m spatial resolution and projected to WGS 84 (EPSG: 4326) to ensure spatial comparability across analytical workflows, following the harmonization approach outlined by Yahia Meddah et al. (2023).

Multi-temporal Landsat 5 TM, 7 ETM+, and 8–9 OLI Surface Reflectance imagery (1995, 2005, 2015, 2025) was acquired through Google Earth Engine (GEE) to derive the Modified Normalized Difference Water Index (MNDWI) and NDVI, supporting shoreline delineation and vegetation assessment (USGS, 2024; Chowdhury & Nayon, 2025) . Elevation and slope were generated from the SRTM DEM (NASA JPL, 2013), while geomorphic characteristics were derived from the ESA WorldCover v200 (2021) product, resampled to match the analysis grid (ESA, 2022) .

A field-calibrated soil salinity raster (2024) and a GEE-derived proximity-to-shoreline layer captured ecological and exposure gradients crucial for coastal vulnerability assessment. Population exposure was represented using WorldPop v2.0 (2020) and resampled from 100 m to 30 m to maintain consistent spatial resolution (WorldPop, 2017) .

All datasets used in the study are summarized in Table 3.1.

Table 3.1. Datasets Used in the Coastal Assessment Workflow

| Variable | Dataset / Source | Dataset ID | Year(s) | Resolution | Application | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| MNDWI | Landsat 5 TM, 7 ETM+, 8–9 OLI SR (via GEE) | LT05, LE07, LC08 | 1995, 2005, 2015, 2025 (Feb) | 30 m | Shoreline extraction | USGS (2024) |
| Erosion–Accretion (Ea) | DSAS-derived LRR | — | 1995–2025 | 30 m | Shoreline change | Chowdhury & Nayon (2025) |
| Elevation (El) | SRTM DEM | USGS/SRTMGL1_003 | 2000 | 30 m | Coastal elevation | NASA JPL (2013) |
| Slope (Sp) | Derived from SRTM DEM (GEE terrain) | — | 2000 | 30 m | Terrain gradient | Islam et al. (2025) |
| Geomorphology (Gm) | ESA WorldCover v200 | ESA/WorldCover/v200 | 2021 | 10 m → 30 m | Landform classification | Zanaga et al. (2022) |
| Vegetation (NDVI) | Landsat 8–9 SR | LANDSAT/LC09/C02/T1_L2 | 2020–2025 | 30 m | Vegetation density | USGS (2024) |
| Soil Salinity (Sy) | Field-calibrated raster | — | 2024 | 30 m | Salinity stress | Islam et al. (2025) |
| Proximity (Pr) | Euclidean distance from shoreline (GEE) | — | 2025 | 30 m | Exposure to coast | Islam et al. (2025) |
| Population (Nd) | WorldPop Bangladesh v2.0 | WorldPop v2.0 | 2020 | 100 m → 30 m | Socioeconomic exposure | WorldPop (2017) |

3.2 Pre-processing (GIS + ML)

All datasets were pre-processed through a combined GIS–DSAS workflow, with additional automated procedures performed in association with Google Earth Engine (GEE) to ensure spatial uniformity and analytical consistency. This dual approach allowed precise shoreline extraction and efficient generation of environmental predictors required for both GIS-based and ML-based analyses.

GIS–DSAS Pre-processing (Traditional)

Multi-temporal Landsat Surface Reflectance imagery (1995, 2005, 2015, 2025) was atmospherically corrected and used to compute the Modified Normalized Difference Water Index (MNDWI) following Chowdhury and Nayon (2025) . Shorelines extracted from MNDWI were analysed using the Digital Shoreline Analysis System (DSAS v6.0) in ArcGIS to derive Linear Regression Rate (LRR) and End Point Rate (EPR) along 100 m transects, incorporating a positional uncertainty of ±20 m (Open-File Report, 2009; USGS) . These DSAS outputs represent the core traditional shoreline metrics used in subsequent GIS and ML components.

GEE-Based Pre-processing (In Association With GIS)

DSAS-derived LRR transects were integrated in association with GEE, where Inverse Distance Weighted (IDW) interpolation was applied across a 2 km coastal buffer to generate a continuous erosion–accretion surface. The interpolated raster was then classified into five rule-based vulnerability levels following Kumar et al. (2010) and Hegde et al. (2019) . Environmental variables were also generated in association with GEE, including elevation and slope from the SRTM DEM (NASA JPL, 2013; Islam et al., 2025), geomorphology from the ESA WorldCover v200 dataset (Zanaga et al., 2022), NDVI composites for 2020–2025 (Uddin et al., 2023), soil salinity from field-calibrated rasters, and proximity to coastline computed via Euclidean distance (Islam et al., 2025) . All rasters were resampled to 30 m, clipped to the study area, and categorized into five vulnerability classes (1–5) using deterministic or quantile-based thresholds depending on variable behavior (Pantusa et al., 2022; Theocharidis et al., 2024a,b).

ML Pre-processing

Machine learning pre-processing focused on converting the DSAS shoreline-change metrics and environmental variables into a unified dataset suitable for modelling. All DSAS outputs (LRR, EPR, NSM, SCE) were spatially aligned with remote-sensing predictors such as NDVI, geomorphology, elevation, slope, and distance-to-shoreline, following the harmonisation standards used in recent shoreline–ML studies (Fogarin et al., 2023).

Continuous predictors were normalised using min–max scaling to improve model stability, while categorical geomorphic classes were numerically encoded, consistent with coastal ANN-based modelling approaches (Peponi et al., 2019). DSAS indicators were retained both as target variables and, where useful, as supplementary features, a strategy also used in long-term shoreline evolution modelling (Eteh et al., 2024).

Imbalanced erosion–accretion classes were handled through stratified sampling to avoid bias, aligning with machine-learning–based coastal vulnerability studies (Ennouali et al., 2023). Outlier shoreline-change values were screened but preserved when linked with real events such as cyclones, reflecting practices in Nile Delta and Indian coastline analyses (El-Asmar and Hereher, 2011) and (Kumar et al., 2020).

The final ML-ready dataset contained normalised continuous variables, encoded geomorphology, environmental indices, cyclone attributes, and DSAS metrics, similar to frameworks used in deep-learning shoreline-change studies in Malaysia and global coasts (Khurram et al., 2025).

3.3 Shoreline Dynamics (GIS + ML + Comparison/Correlation)

Shoreline dynamics were assessed using a combined traditional GIS–DSAS workflow supported in association with GEE, alongside ML-based shoreline interpretation and classification. This integrated design enabled the extraction of decadal shoreline movement, identification of spatial patterns, and preparation of derived metrics for subsequent ML modeling.

GIS–DSAS Shoreline Analysis (Traditional)

Four decadal shorelines (1995, 2005, 2015, 2025) were delineated from February Landsat imagery using the MNDWI method following Chowdhury and Nayon (2025) . The extracted shorelines were analyzed in DSAS v6.0 to compute Linear Regression Rate (LRR) and End Point Rate (EPR) along 100 m transects, accounting for positional uncertainty of ±20 m (Open-File Report, 2009; USGS). These DSAS-derived indicators provided the baseline erosion–accretion metrics representing shoreline stability across the study period.

Spatial Refinement in Association with GEE (Modern GIS)

The LRR transect outputs were processed in association with GEE, where a 2 km IDW interpolation produced a continuous erosion–accretion raster at 30 m resolution, following the spatial refinement methodology reported by Goswami et al. (2022) . The interpolated surface was then reclassified into five vulnerability classes using physically meaningful thresholds adopted from Kumar et al. (2010) and Hegde et al. (2019), enabling consistent integration into downstream CVI and ML models.

ML-Based Shoreline Dynamics (NSM, SCE, EPR, LRR)

Machine learning was used to interpret and classify the four DSAS-derived shoreline indicators Net Shoreline Movement (NSM), Shoreline Change Envelope (SCE), End Point Rate (EPR), and Linear Regression Rate (LRR) to identify spatial patterns of coastal stability, active erosion, and accretion. These metrics quantify both the magnitude and the rate of shoreline shift (Weerasekara and Illeperuma, 2024) and therefore serve as suitable targets for ML-based shoreline analysis.

Target Variable Construction

The NSM, SCE, EPR, and LRR values computed along each DSAS transect were directly compiled from the pre-processed dataset. NSM and SCE capture the total distance of shoreline movement across years, while LRR and EPR represent annualised rates of change (Weerasekara and Illeperuma, 2024). These four indicators were treated either as continuous targets for regression or discretised into erosion–stable–accretion classes for ML classification, depending on the model used in the notebook.

Model Training and Pattern Learning

ML models (including Random Forest, Gradient Boosting, and related ensemble regressors) were trained on a feature matrix comprising NDVI, elevation, slope, geomorphology, salinity, proximity to shoreline, and interpolated erosion–accretion surfaces. These environmental predictors are known to influence shoreline behaviour and have been successfully used in previous shoreline-ML studies (Khurram et al., 2025; Park and Song, 2024).

The models learned characteristic patterns, such as:

- High NSM/SCE magnitudes associated with low-elevation, high-exposure zones, patterns also observed in multi-decadal analyses in Malaysia (Khurram et al., 2025).

- Negative LRR/EPR values concentrated in erosion-dominated sectors, consistent with documented DSAS behaviour (Weerasekara and Illeperuma, 2024).

- Positive LRR/EPR values in accreting sectors influenced by sediment supply or depositional processes, aligning with global shoreline trends described in (Kumar et al., 2020).

Shoreline Stability Classification

Using class-labelled versions of NSM, SCE, EPR, and LRR, the ML models classified each transect into Erosion (negative change), Stable (near-zero change), Accretion (positive change)

Stability zones identified by the ML workflow corresponded closely with known stable transect distributions documented in DSAS-based long-term shoreline assessments (Khurram et al., 2025). High-risk erosion zones were consistently detected where NSM and LRR showed large negative excursions, matching analytical trends reported in DSAS literature (Weerasekara and Illeperuma, 2024).

ML-Enhanced Spatial Understanding

The ML predictions were overlaid with DSAS results to highlight convergence zones where both approaches agreed, and divergence zones where ML detected subtle morphodynamic patterns not fully captured by traditional linear regression. Such integration of deterministic DSAS outputs with ML insights aligns with emerging hybrid methodologies for shoreline analysis (Khurram et al., 2025).

GIS–ML Comparison and Correlation

The spatial correspondence between DSAS-derived LRR/EPR and ML-predicted shoreline classes was evaluated through direct overlay and statistical comparison to identify agreement and divergence zones. This correlation step allowed the integration of deterministic GIS indicators with data-driven ML outputs, forming a coherent basis for stability analysis, hotspot detection, and subsequent CVI modeling.

3.4 CVI Modeling (GEE + ML + Comparison)

Coastal vulnerability in Kuakata was assessed through a fully integrated modeling framework that combined a physically interpretable GIS–GEE CVI, a statistical quantile-based CVI, and a machine-learning–enhanced vulnerability model. Each component contributed a different analytical strength: the rules-based CVI captured deterministic geomorphic principles; the quantile model offered statistical balance; and ML uncovered deeper sediment dynamics, stability patterns, and spatial hotspots. Together, they formed a coherent multi-method structure capable of describing both the process-driven and data-driven dimensions of coastal vulnerability.

3.4.1 CVI Variables

Seven variables were used to represent the physical and environmental conditions that control coastal vulnerability: Erosion–Accretion (Ea), Elevation (El), Slope (Sp), Geomorphology (Gm), Vegetation (NDVI), Soil Salinity (Sy), and Proximity (Pr). Each dataset was selected for its established relevance in global CVI research and processed at 30 m resolution for uniformity.

The erosion–accretion parameter (Ea) originated from DSAS-derived LRR values and was subsequently transformed into a continuous surface in association with GEE, leveraging its cloud-based interpolation capabilities (Goswami et al., 2022) . Elevation and slope came from the SRTM DEM (NASA JPL, 2013), vegetation was represented by Landsat 8–9 NDVI composites (Uddin et al., 2023), geomorphic classes were based on ESA WorldCover v200 (Zanaga et al., 2022), salinity was derived from field-calibrated rasters, and proximity measured Euclidean distance from the shoreline using GEE (Islam et al., 2025).
All variables were reclassified into five ordered vulnerability levels following Pantusa et al. (2022) and Theocharidis et al. (2024a,b) to ensure consistency across the modeling framework.

Table 3.4.1. CVI and ML Variables with Data Sources and Analytical Roles

| Variable | Source Dataset | Derivation Method | Analytical Role (GIS/GEE) | Analytical Role (ML) | Reference |
| --- | --- | --- | --- | --- | --- |
| Ea (Erosion–Accretion) | DSAS LRR/EPR + Landsat SR | DSAS transects → IDW interpolation (in association with GEE) | Core geomorphic indicator | Target + predictor for ML shoreline classes | Chowdhury & Nayon (2025); Goswami et al. (2022) |
| El (Elevation) | SRTM DEM | GEE terrain function | Topographic constraint | Key predictor for ML stability, sediment dynamics | NASA JPL (2013) |
| Sp (Slope) | SRTM DEM | Derived in association with GEE | Coastal steepness indicator | ML gradient-related erosion predictor | Islam et al. (2025) |
| Gm (Geomorphology) | ESA WorldCover v200 | Reclassified to coastal landforms | Surface stability & exposure | Encoded ML feature for terrain behaviour | Zanaga et al. (2022); Achmakidou et al. (2025) |
| Nd (NDVI) | Landsat 8–9 SR | Median NDVI composites | Vegetation protection proxy | ML predictor for sediment resistance | Uddin et al. (2023) |
| Sy (Soil Salinity) | Field-calibrated raster | Normalized ranges | Ecological stress indicator | ML feature for coastal degradation | 2009_03_13_estuaries_monitor_chap14 |
| Pr (Proximity) | Derived shoreline distance | Euclidean distance (GEE) | Exposure to marine forcing | ML predictor for erosion hotspots | Islam et al. (2025) |
| Population density | WorldPop v2.0 | Resampled to 30 m | CRI exposure component | ML risk and hotspot modeling | WorldPop (2017) |
| NSM, SCE, EPR, LRR | DSAS outputs | Transect-based metrics | GIS shoreline change diagnostics | ML target classes for shoreline dynamics | Open-File Report (2009); Minh Quan et al. (2025) |

3.4.2 Rules-Based vs Quantile-Based CVI (With Comparison)

Two CVI models were constructed to assess how classification logic influences vulnerability outcomes and to ensure methodological robustness.

Rules-Based CVI

The rules-based CVI applied physically meaningful thresholds that directly reflect coastal processes. Variables such as low elevation (<2 m), gentle slope (<1°), tidal flats, bare beaches, low vegetation, high salinity, and intense erosion were systematically assigned higher vulnerability classes, following the criteria described by Theocharidis et al. (2024b), Das et al. (2019), and Pantusa et al. (2022) . Each rank was converted into a standardized vulnerability score:

Si={0.05,  0.30,  0.60,  0.80,  1.00}………(3.4.1)

ensuring that physically meaningful categories translated into continuous measures suitable for aggregation.

Quantile-Based CVI

To provide a statistically balanced alternative, a mixed quantile-based CVI classified elevation, slope, geomorphology, and proximity into 20% percentile intervals, ensuring that each class contained an equal proportion of pixels (Achmakidou et al., 2025b; Hamid et al., 2019) . The CVI was computed using the multiplicative form:

CVIquantile=7Ea×El×Sp×Gm×Nd×Sy×Pr………(3.4.2)

and normalized by:

CVInorm=CVI-CVIminCVImax-CVImin………(3.4.3)

3.4.3 CVI Detection Enhanced by ML Outputs

The ML-identified hotspots were integrated with the Coastal Vulnerability Index (CVI) layers to refine vulnerability detection. While the GIS-based CVI relies on deterministic ranking of factors such as elevation, slope, geomorphology, NDVI, salinity, and erosion–accretion (Pantusa et al., 2022; Theocharidis et al., 2024), the ML models provided an additional layer by detecting subtle combinations of predictors that statistically correlate with shoreline instability.

In several coastal studies, ML has proven effective at revealing risk clusters not obvious in traditional CVI surfaces because of its ability to learn complex interactions between environmental variables (Adeli et al., 2025).

3.4.4 Integrated CVI (CRI)

The Coastal Risk Index (CRI) extends the rules-based CVI by incorporating population exposure to reveal where physical vulnerability and human presence converge along the Kuakata coast. This integration is critical in deltaic environments where geomorphic instability often coincides with dense settlement patterns. The CRI thus transforms coastal vulnerability from a purely environmental measure into a socially relevant, risk-oriented index, consistent with the framework presented by Field et al. (2012) and applied across coastal risk studies such as Tanim et al. (2022) .

(a) Data Integration and Analytical Framework

CRI development relied on two harmonized datasets. First, the rules-based CVI, constructed from seven geophysical variables—Erosion–Accretion (Ea), Elevation (El), Slope (Sp), Geomorphology (Gm), NDVI (Nd), Soil Salinity (Sy), and Proximity (Pr) each classified into five physically meaningful vulnerability ranks (Open-File Report, 1999) .

Second, a population density layer, derived from WorldPop, was similarly ranked into exposure classes representing increasing settlement intensity.

Both layers were resampled to 30 m spatial resolution and projected to WGS 84, ensuring pixel-level alignment for risk computation.

(b) Risk Calculation and Normalization

Raw coastal risk was calculated by multiplying the CVI and population exposure layers (Tanim et al., 2022):

CRIraw=CVI×Population(3.4.4)

This formulation captures the interaction between environmental hazard and human presence, producing higher scores where both dimensions are elevated.
CRI_raw was normalized using min–max scaling (Akter & Ataullah, 2024):

CRInorm=CRIraw-CRIminCRImax-CRImin(3.4.5)

This yielded a continuous 0–1 index suitable for classification and spatial interpretation.

(c) Rules-Based Classification

The normalized CRI was translated into five deterministic classes—very low to very high risk—using fixed thresholds consistent with the rules-based CVI and established coastal risk frameworks (Thieler & Hammar-Klose, 1999; Gopinath et al., 2023; Hamid et al., 2019) .

Table 3.4.3. CRI Classification Scheme

| Class | CRI Range | Risk Level | Description |
| --- | --- | --- | --- |
| 1 | 0.00–0.20 | Very Low | High elevation or sparse population; minimal exposure |
| 2 | 0.20–0.40 | Low | Moderate elevation; limited settlement concentration |
| 3 | 0.40–0.60 | Moderate | Transitional inland belt; mixed exposure |
| 4 | 0.60–0.80 | High | Populated coastal settings near vulnerable shorelines |
| 5 | 0.80–1.00 | Very High | Highly exposed, densely settled coastal fringes |

This deterministic scheme avoids the artificial smoothing produced by quantile-based classifications and ensures that risk classes retain clear physical meaning.

(d) Integrated CVI-ML Framework

The CVI ranks each variable using physically meaningful coastal thresholds, while the ML components highlight locations where statistical patterns indicate elevated shoreline instability. This dual-input structure aligns with emerging risk-mapping practices where machine learning is used to supplement deterministic coastal indicators (Adeli et al., 2025) and multidecadal shoreline-instability analyses (Khurram et al., 2025).

(d) Spatial Patterns and Interpretation

The CRI map revealed a distinct coastal-to-inland risk gradient across Kuakata.
The highest risk zones (classes 4–5) were sharply concentrated along low-lying, erosion-dominated sectors near Kuakata town and adjacent fishing-tourism settlements. These hotspots represent the convergence of geomorphic instability—low elevation, shallow slope, tidal flats, salinity intrusion and intense human occupation, reflecting patterns described by Al-Mannai et al. (2025) and Krishnan et al. (2019) .

Moderate-risk areas (class 3) extended inland through transitional geomorphic belts with mixed land cover and intermediate population density. Low and very-low risk zones (classes 1–2) occupied elevated or vegetated inland patches where both hazard and exposure were limited.

These spatial patterns reinforce a critical insight: population density magnifies coastal risk, elevating areas that may appear only moderately vulnerable when assessed through physical indicators alone—a phenomenon also highlighted by Xu et al. (2022) .

(e) Final Model Adoption and Policy Relevance

The rules-based CRI was adopted as the final operational model because it produced clear, physically grounded, and spatially coherent risk patterns that aligned strongly with field-observed coastal processes (Pantusa et al., 2022) . In comparison, the quantile-based CRI—although statistically balanced—tended to mask critical high-risk areas due to distribution-driven smoothing, similar to limitations noted by Minh Quan et al. (2025) .

The finalized CRI offers a robust, decision-ready framework for identifying priority mitigation zones, guiding disaster preparedness, and supporting long-term climate adaptation planning across the dynamic deltaic landscape of Kuakata.